DOSSIER TECHNIQUE

1	STADE EAU VIVE	2

2	PRESENTATION  DU STADE D’EAU VIVE	3

2.1	Le site :	3

2.2	Description succincte du cycle de fonctionnement :	3

2.3	Infrastructure:	3

2.4	Site à marée montante :	4

2.5	Site à marée descendante :	5

2.6	Vue détaillée du stade :	6

2.7	Profil de la réserve et de la rivière artificielle	6

2.8	Vue détaillée de la vanne omniflots	7

2.8.1	En  position intermédiaire :	7

2.8.2	Totalement relevée :	7

2.8.3	Totalement affalée :	8

3	ARCHITECTURE DU SYSTEME INFORMATIQUE	8

4	LES MAREES	9

4.1	Définition de la Marée	9

4.2	Origine de la marée	9

4.3	Marée de vive-eau, marée de morte-eau	10

4.4	Le marnage	11

4.5	Le coefficient de marée	12

4.6	Exemple de marée	12

4.7	Exemple de calcul de marées	13

5	DOCUMENTATIONS TECHNIQUES	14

5.1	Noyau temps réel  NTR++	14

5.1.1	les Threads	14

5.1.2	Les Sémaphores	15

5.1.3	les Evénements	16

5.1.4	Exemple d’utilisation des objets NTR++	17

5.2	Le protocole HTTP 1.0	18

5.2.1	Présentation	18

5.2.2	Requête HTTP	18

5.2.3	Réponse HTTP	19

5.2.4	Http et CGI	21

5.3	Carte Lx162	22

5.4	IPModule	24

5.4.1	introduction	24

5.4.2	IP-Modules	24

5.4.3	Bus description	25

5.5	Controleur  IPIC	26

5.5.1	Features	26

5.5.2	Clock	26

5.5.3	Interrupts	26

5.5.4	Memory Map	26

5.5.5	Programming Model	27

5.6	Module ADA08	30

Nota :   Pour l’examen, le candidat devra être en possession de ce dossier et  de ses supports de cours sur:

- Le langage C++,

- Le langage UML (diagrammes de collaborations, séquences et classes),

- Les protocoles réseaux (Ethernet, IP, UDP, TCP, BOOTP).

# STADE EAU VIVE

Au fond d’une petite baie située sur la Manche, les phénomènes de marée sont utilisés pour alimenter une rivière artificielle à débit variable. Des obstacles mobiles permettent de modifier à loisir le tracé de cette rivière longue de 300 mètres.

Cet ensemble constitue donc un Stade d’Eau Vive  où petits et grands peuvent découvrir des activités en toute sécurité : raft, mini-raft, canoë gonflable, kayak ;

Ce Stade d’Eau Vive peut se transformer en:

- un bassin d'initiation pour apprendre avec un moniteur à maîtriser les mouvements de l'eau ;

- un bassin de perfectionnement puisque le niveau de difficulté peut augmenter en fonction des progrès des utilisateurs ;

- un bassin d'entraînement, où les kayakistes de niveau régional, national ou international, peuvent s'entraîner toute l'année dans des conditions semblables à celles offertes par les torrents de montagne.

De l'amateur au sportif accompli, chacun peut donc profiter des plaisirs de l’eau vive au rythme des marées .

# PRESENTATION  DU STADE D’EAU VIVE

## Le site :

Le site se décompose en trois parties :

- La baie qui sert de réserve d’eau.

- Le barrage constitué, d’une digue en béton et de 2 vannes, qui ferme la baie.

- La rivière artificielle, alimentée par l’eau de la réserve, qui comporte des obstacles reconfigurables et qui permet les entraînements ou les compétitions de kayak ou canoë. Cette rivière artificielle a une longueur de 300 m et présente une pente de 0,5%, elle est totalement recouverte d’eau à marée haute et n’est donc utilisable qu’à certaines heures de la marée (quand elle émerge totalement).

## Description succincte du cycle de fonctionnement :

- Pendant la marée montante, les vannes ouvertes permettent le remplissage de  la réserve.

- A pleine mer les vannes sont fermées.

- Pendant la marée descendante la rivière artificielle émerge progressivement. Elle devient utilisable quand le niveau de la mer atteint le point le plus bas du stade.

- Il est alors possible d’ouvrir une des vannes du barrage afin d’alimenter en eau la rivière artificielle et ceci avec un débit contrôlé.

- A partir de ce moment la rivière artificielle peut être utilisée par les kayakistes.

- Lorsque la réserve est épuisée ou que la durée maximum d’utilisation est atteinte, les vannes sont affalées pour vider l’eau résiduelle (lutte contre l’envasement) et préparer le remplissage suivant.

Les cycles se succèdent ainsi marée après marée.

## Infrastructure:

La réserve est constituée d’un bassin que l’on assimilera à un parallélépipède rectangle de surface 120000 m² (2000 m de long sur 60 m de large)

Le barrage qui ferme la baie est équipé :

- d’une vanne de 7 m de large destinée à alimenter la rivière artificielle, cette vanne sera appelée dans la suite du sujet : vanne omniflots

- d’une vanne de 20 m de large destinée à assurer un remplissage rapide de la réserve. Cette vanne sera appelée dans la suite du sujet : vanne stockVide

Les schémas suivants représentent :

- le site à marée descendante (vues de dessus et en coupe)

- le site à marée montante  (vues de dessus et en coupe)

- une vue détaillée du stade

- le profil de la réserve et de la rivière artificielle

- trois vues de détail de la vanne omniflots (positions : fermée, ouverte et intermédiaire)

## Site à marée montante :

## Site à marée descendante :

## Vue détaillée du stade :

## Profil de la réserve et de la rivière artificielle

Le niveau NGF est utilisé sur les cartes terrestres, il correspond au niveau de mi-marée (ici 5m hydrographique = 0m NGF).

## Vue détaillée de la vanne omniflots

### En  position intermédiaire :

0

0,5

1

1,5

2

2,5

3

3,5

4

-2

-1

1

2

3

4

5

niveau NGF en m

wwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwww

Le sommet de la  vanne décrit un arc de cercle

Sens d’écoulement de l’eau pendant l’utilisation du stade.

Vers la mer

Coté réserve

Fond du stade, la pente a été exagérée pour clarifier le schéma.

Fond de la réserve

radier

la différence de niveau entre l’eau de la réserve et le sommet de la vanne détermine le débit de l’eau dans le stade.

wwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwwww

distance en m

Toit de la vanne, il   guide l’eau et permet un passage aisé des kayaks

### Totalement relevée :

### Totalement affalée :

# ARCHITECTURE DU SYSTEME INFORMATIQUE

Le site comporte principalement les matériels informatiques suivants :

- Un micro ordinateur industriel comprenant une carte VME Motorola LX 162 et des modules d’entrées sorties Industry Pack.

- Un terminal X dédié à l’interfaçage homme machine.

Au niveau logiciel :

- la carte LX 162 est munie, en mémoire flash :

- d’un noyau temps réel NTR++ ,

- d’une agence réseau TCP/IP,

- d’un serveur WEB embarqué.

- le terminal X possède (en mémoire Flash) :

- un serveur Xwindows,

- deux clients locaux (Telnet + navigateur Web) ;

# LES MAREES

Tous ceux qui fréquentent le littoral français ont observé les effets de la marée sur nos côtes ; ce phénomène influence l'environnement et la vie quotidienne de milliers de personnes : des marins aux simples baigneurs...

Peu sensibles en Méditerranée (environ 40 cm de marnage), les variations régulières du niveau de la mer sont plus importantes sur la façade Atlantique et en Manche où elles peuvent dépasser les 10 mètres (jusqu'à 14 mètres en baie du Mont-Saint-Michel).

## Définition de la Marée

La marée est la variation du niveau de la mer due à l'action gravitationnelle de la Lune et du Soleil, astres dont les mouvements peuvent être calculés avec précision sur des périodes de plusieurs centaines, voire de plusieurs milliers d'années. L'un des buts principaux de l'étude des marées est la recherche des relations existant entre le mouvement des astres et la réponse des océans à l'action de ces forces gravitationnelles afin d'établir des formules de prédiction. A ces mouvements d'allure régulière se superposent des variations de hauteur d'eau d'origine météorologique, appelées surcotes-décotes, dont l'étude relève essentiellement de méthodes statistiques.

## Origine de la marée

La marée astronomique est une manifestation de la loi de la gravitation universelle appliquée au système formé par la Terre, le Soleil et la Lune.

La force génératrice de la marée est la résultante de deux forces :

- la force d'attraction gravitationnelle exercée par l'astre, proportionnellement à sa masse et en raison inverse du carré de sa distance

- la force centrifuge identique en tout point de la Terre, due au mouvement de la Terre sur son orbite autour du centre de gravité du système Terre-astre.

Au centre de la Terre, ces deux forces se compensent exactement.

Lorsque l'astre est au-dessus de l'horizon, la force d'attraction de l'astre est la plus importante.

Lorsque l'astre est au-dessous de l'horizon, c'est la force centrifuge qui l'emporte.

Si l'océan était en équilibre avec la force génératrice de la marée, sa surface prendrait la forme d'un ellipsoïde de révolution dont le grand axe serait dirigé vers l'astre. Ce phénomène a reçu le nom de marée statique.

Du fait de la rotation de la terre autour de son axe, un observateur situé à sa surface observerait généralement deux pleines mers et deux basses mer par jour, l'une dans la direction de l'astre, l'autre dans la direction opposée, deux basses mers étant observées lorsque l'astre est à l'horizon.

Il peut arriver, pour les latitudes élevées, lorsque la déclinaison est importante, que l'astre n'atteigne pas l'horizon. La pleine mer secondaire a alors disparu et la marée devient de type diurne : on observe une seule pleine mer et une seule basse mer par jour.

## Marée de vive-eau, marée de morte-eau

La marée étant générée par la Lune et le Soleil, les actions de ces deux astres peuvent s'ajouter ou se contrarier selon leurs positions relatives.

Les variations de hauteur d'eau sont conditionnées par les phases de la Lune .

|  | Les trois astres sont alignés ; les forces s'additionnent ; les marées sont importantes. |
| --- | --- |
|  | Les trois astres forment un angle droit ; les forces se contrarient ; les marées sont faibles. |
|  | Les trois astres sont alignés ; les forces s'additionnent ; les marées sont importantes. |
|  | Les trois astres forment un angle droit ; les forces se contrarient ; les marées sont faibles. |

- Lorsque le marnage (dénivellation entre la pleine mer et la basse mer) passe par un maximum, la marée est dite de vive-eau. Elle correspond aux phases de nouvelle lune et de pleine lune appelées syzygies. Elle s'explique par les effets conjugués de la Lune et du Soleil.

- Lorsque le marnage passe par un minimum, la marée est dite de morte-eau. Elle correspond aux phases de premier et de dernier quartiers de la Lune, appelées quadratures. Elle s'explique par les effets opposés de la Lune et du Soleil.

Donc, à chaque pleine lune et à chaque nouvelle lune, environ tous les quinze jours, les amplitudes de marée passent par un maximum. A chaque premier quartier et dernier quartier, les amplitudes de marée passent par un minimum.

Les vives-eaux et mortes-eaux interviennent avec un certain retard par rapport aux syzygies et aux quadratures. Ce retard est l'âge de la marée.

Vives-eaux et mortes-eaux

## Le marnage

Le marnage est la différence entre une pleine mer et une basse mer successives. Le marnage varie selon la période de vive-eau, moment où il est plus fort et la période de morte-eau où, à contrario, il est plus faible. Les marnages peuvent être très différents d'une zone à l'autre. Le marnage maximal observé dans le monde est dans la baie de Fundy , au Canada entre la Nouvelle-Écosse et le Nouveau-Brunswick. Le marnage peut y atteindre jusqu'à 16m.

En France, dans la baie du Mont-Saint-Michel, le marnage peut atteindre jusqu'à 14m par forts coefficients.

## Le coefficient de marée

En France, on associe à l'amplitude de l'oscillation de la marée un coefficient dit coefficient de marée.

La prédominance de la marée semi-diurne sur les côtes de La Manche et de l'Atlantique confère au coefficient de marée de Brest son caractère général, valable sur les côtes Atlantique et Manche. Il permet de façon simple et rapide de connaître l'importance de la marée un jour donné et en particulier les dates des grandes marées.

C'est un nombre sans dimension compris entre 20 et 120 et qui varie peu d'un jour sur l'autre. Par convention, le coefficient 100 est attribué au marnage semi-diurne moyen lors des vives-eaux voisines des équinoxes (21 mars, 21septembre).

* marées extraordinaires de vive-eau d'équinoxe; le marnage à Brest est admis à 7,32m d'où le coefficient maximal de 120
* marées de vive-eau moyenne : 95
* marées moyennes : 70
* marées de morte-eau moyenne : 45
* marées de morte-eau les plus faibles; le marnage à Brest est admis à 1,22 m, d'où le coefficient minimal de 20.

## Exemple de marée

| La marée du jour (vendredi 10 septembre 1999) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Matin |  |  |  |  |  |  |  |  | Après-midi |  |  |  |  |  |  |  |  |
| Coeff. | Pleine mer | Hauteur | Basse mer | Hauteur | Coeff. | Pleine mer | Hauteur | Basse mer | Hauteur |
| 97 | 7h29 | 9,21m | 1h32 | 1,30m | 98 | 19h42 | 9,49m | 13h48 | 1,36m |

Marégramme : graphique représentant les variations du niveau de la mer  en fonction du temps.

Zéro hydrographique

0

Niveau de mi-marée = 0 NGF

Basse mer (BM) : niveau le plus bas atteint par la mer au cours d'un cycle de marée.

Pleine mer (PM) : niveau le plus élevé atteint par la mer au cours d'un cycle de marée.

Zéro hydrographique ou zéro des cartes marines : Niveau de référence à partir duquel sont comptées positivement les hauteurs de marée. En France, le zéro hydrographique est voisin du niveau des plus basses mers théoriques.

Niveau de mi-marée: Hauteur moyenne de la surface de la mer calculée pour l'ensemble des stades de la marée (mortes-eaux et vives-eaux) et qui est utilisée comme référence pour la mesure de l'altitude terrestre. C’est le zéro des cartes terrestres (NGF ).

## Exemple de calcul de marées

Avant la construction d’un pont en 1971, l’île de Noirmoutier était uniquement reliée au continent par une chaussée pavée, longue de 4,5 km, découvrant à marée basse: le passage du Gois.

Cette chaussée, utilisable deux fois par jour pendant quelques heures, donnent des frayeurs à certains automobilistes imprudents, qui passant trop tard à marée montante, doivent abandonner leur véhicule et grimper sur une balise refuge s’ils ne veulent pas être emportés par la mer.

En 1997, pour sécuriser son utilisation, de nouveaux panneaux automatiques indiquant les horaires des marées ont été mis en service aux abords du Gois, côté île et côté continent.

Le 5 juillet 1999, les coureurs du Tour de France empruntent ce passage pour rejoindre l’île de Noirmoutier. Les organisateurs de cette épreuve ont dû tenir compte des horaires de la marée pour assurer le passage des concurrents en toute sécurité.

| La marée du lundi 5 juillet 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Matin |  |  |  |  |  |  |  |  | Après-midi |  |  |  |  |  |  |  |  |
| Coeff. | Basse mer | Hauteur | Pleine mer | Hauteur | Coeff. | Basse mer | Hauteur | Pleine mer | Hauteur |
| 63 | 2h40 | 1,39m | 8h30 | 4,31m | 60 | 15h03 | 1,67m | 21h17 | 4,47m |

Sachant que le niveau de mi-marée pour ce lieu est de 3m hydrographique et que le passage du Gois  est à une altitude constante d’environ 0m NGF :

- A quelle heure de l’après midi, les services techniques pourront ils commencer le nettoyage (algues, sable, gravillons, ...)  de la chaussée du Gois ?

- Avant quelle heure la voiture balai, qui ferme la marche du tour, doit-elle emprunter ce passage ?

- Les organisateurs veulent une marge de 50 cm entre le niveau de la chaussée et le niveau de la mer quand les coureurs emprunteront ce passage. Quelle est la marge de temps (écart entre horaire maxi et horaire mini) disponible.

- Un voilier ayant un tirant d’eau de  1 m se présente à 11h pour franchir le Gois. Vu le manque d’eau, son skipper décide de se laisser échouer et d’assister au passage du tour. A quelle heure pourra t’il reprendre sa route pour franchir le passage ?

Réponses:  	a)   12h15 		b)   17h30		c)   4 h (13h à 17h)		d)   19h15

# DOCUMENTATIONS TECHNIQUES

## Noyau temps réel  NTR++

La carte Lx 162 est équipée d’un noyau temps réel objet multi-threads romable NTR++. Les classes  Thread, Semaphore et Event sont  utilisables. On trouvera ci-dessous le fichier d’entête correspondant ainsi que la description UML.

### les Threads

/***************************************************************************

*	Module Name:		Thread.h

****************************************************************************/

#ifndef _THREAD_H

#define _THREAD_H

/*--  Class Thread is the basic class for NTR++ thread control.

Thread

+  Thread(startRoutine : void (*) (void), priority : int = 10) : Thread

#  pri : int

#  idThread : int

+  ~Thread ()

+  suspend() : void

+  resume() : void

+  cancel() : void

+  terminate() : void

+  sleep(sec : long) : void

+  tickSleep(ticks : long) : void

Usage example:

#include <Thread.h>

void threadEntryPoint( ) ;

void main( )

{  ….

Thread first(threadEntryPoint);

Thread second(threadEntryPoint, 50);

….

first.cancel( ) ;

second.cancel( ) ;

}

--*/

class Thread {  	 // The scheduler is based on the highest-priority ready task determined by :

// 1) the priority level you assign to the task when it is created

// 2) the order tasks are made ready among equal-priority tasks

public:	//  create and start a thread with    1(low) =<priority =<255(high)   ( default=10)

Thread (void  (*startRoutine) ( ), int   priority = 10);

~Thread ( );				// delete the thread

void	suspend ( );			// suspend thread

void	resume ( );	                	// resume thread

void	cancel ( );	                	// cancel thread

static void	terminate ( );           		// terminate current thread

static void	sleep (long sec);           		// delay current thread

static void	tickSleep (long ticks);		// sleep in timer tick units (1/100e s)

protected:

int	pri;			// thread priority

int	idThread;		// thread id

};

#endif /* _THREAD_H */

### Les Sémaphores

/****************************************************************************

*	Module Name:		Semaphore.h

*

****************************************************************************

*/

#ifndef _SEMAPHORE_H

#define _SEMAPHORE_H

/*--

Class Semaphore is interface to the NTR++ semaphore object.

Usage example:

#include <Semaphore.h>

Semaphore s; // binary semaphore

s.wait();

// critical section code here

s.post();

Semaphore sem(2); // general semaphore,

// can synchronize access to two resource units

Semaphore

#  idSem : int

-  value : int

+ Semaphore(init_value : int = 1) : Semaphore

+ post() : void

+ ~Semaphore()

+ wait() : void

+ trywait() : int

+ getvalue() : int

sem.wait();     // get one of the two resource units

// utilize the resource

sem.post();

--*/

class Semaphore {

public:

Semaphore (int init_value = 1);

~Semaphore ();

void post ();			// increment semaphore  V(s)

void wait ();			// decrement semaphore (with blocking if sem=0) P(s);

int trywait ();			// if sem >0 decrement semaphore P(s) and return 0

// if sem =0 return ER_NMP without blocking

int getvalue ();		// return semaphore value

protected:

int  idSem;			 // semaphore id

private:

int  value;			 // semaphore value

};

#endif /* _SEMAPHORE_H */

### les Evénements

/***************************************************************************

*

*	Module Name:		Event.h

*

****************************************************************************

*/

#ifndef _EVENT_H

#define _EVENT_H

/*--  Event class is interface to the NTR++ events

Usage example:

#include <Event.h>

Event  e1, e2;

void codeThreadOne( )		void codeThreadTwo( )

{…..				{…..

e1.set( );			 e1.wait( ) ;

…				e1.clear( ) ;

e2.wait( ) ;			…

e2.clear( ) ;			e2.set( );

….				….

}				}

Event

#  idEvent : int

+  Event() : Event

+  ~Event()

+  set() : void

+  clear() : void

+  wait() : void

+  read() : int

--*/

class Event  {

public:

Event ();

~Event ();

void	set ();		// set event

void	clear ();	// clear event

void	wait ();		// wait event

int	read ();		// return state of event

protected:

int idEvent;

};

#endif /* _EVENT_H */

### Exemple d’utilisation des objets NTR++

/***************************************************************************

*	Module Name:		exempleBidon.cpp

****************************************************************************/

#include <Thread.h>

#include <Event.h>

#include <Semaphore.h>

///////////////////////////////////////////////////////////////////////////

// Definition d'une classe chantier

///////////////////////////////////////////////////////////////////////////

class Chantier {

Semaphore brouette(2);

Event auSecours, secourir;

Thread *chef;

public:

//----------------- Constructeur de chantier ------------------------

Chantier ( )		{ chef = new Thread(travailDeChef);}

//------------------ Destructeur de chantier ------------------------

~Chantier( )		{ delete chef;}

//------------------ Travail d'un manoeuvre ---------------------------

void travailManoeuvre( ) 	{

while (1)  {

brouette.wait( );		// prendre une brouette disponible

for (int nbFois=0; nbFois <10; nbFois ++) {

……		 // remplir la brouette de sable …

if (KO)	{ auSecours.set( );	// appel au secours

secourir.wait( );	// attendre secours

secourir.clear( );

}

}

brouette.post( );			// libérer la brouette

Thread::sleep(10*60);   		// repos de 10 mn bien mérite

}

}

//------------------ Travail d'un secouriste ---------------------------

void travailPompier( )  {

while (1) {

auSecours.wait( );		// attente d'un appel

auSecours.clear( );		// acquitter appel

secourir.set( );			// ranimer victime

}

}

//------------------ Travail du Chef         ---------------------------

void travailDeChef( )  {

Thread victor(travailPompier);		//embauche  d'un secouriste

{// Creation de 3 Threads

Thread albert(travailManoeuvre);		// embauche de 3 ouvriers

Thread julien(travailManoeuvre);

Thread emile(travailManoeuvre);

Thread::sleep(8*3600);     		// le chef dort toute la journee

} // fin de bloc :  les 3 Threads ouvriers de classe automatique meurent.

victor.cancel( ) ;		// mort de victor

Thread : : terminate( );  	// le chef se suicide !

}

};  // fin Chantier

// Exemple d'utilisation de la classe Chantier

void main (void) {

Chantier  maMaison;

…..

}

## Le protocole HTTP 1.0

L'HyperText Transfer Protocol (HTTP) est un protocole de niveau application, léger et efficace, pour la transmission de documents distribués et multimédia. Le protocole HTTP 1.0 est décrit dans la RFC 1945. Le résumé ci-dessous est une libre inspiration de différents documents disponibles sur le réseau Internet. Il est forcément incomplet et peut comporter certaines imprécisions.

### Présentation

Ce protocole fixe le principe de communication entre un logiciel de navigation Web (communément appelé "navigateur" ou « browser » ou « client » ) et un serveur Web (souvent appelé httpd , d pour daemon sous Unix ).

Le principe de communication est  simple :

- La communication s'initie TOUJOURS à la demande du client. Celui ci s'adresse à un serveur caractérisé par une adresse IP  ou son nom, sur un port connu, le plus souvent le port 80, mais cela n'a rien d'obligatoire.

- Une connexion TCP s'établit.

- Le client envoie une requête au serveur et ce dernier lui renvoie une réponse correspondant à sa requête.

- Le serveur clôt la connexion une fois la totalité de la réponse émise.

Nous allons maintenant détailler une requête et une réponse HTTP 1.0.

### Requête HTTP

Le format d'une requête HTTP est le suivant :

Ligne de commande

En-tête de la requête

[ligne vide]

Corps de la requête

- La ligne de commande possède elle même trois champs : commande, URL  et version.

- Le premier champ, commande, contient une des commandes définies dans le protocole HTTP. Les principales commandes sont les suivantes :

- GET : demande au serveur de renvoyer le contenu de l'information pointée par l'URL spécifiée dans la ligne de commande. Il peut s'agir d'un simple fichier HTML ou multimédia (image, son, …), voire le compte rendu de l’exécution d'un programme CGI.

- HEAD : cette commande est similaire à la précédente mais le serveur ne renvoie que l'en-tête associé à la ressource demandée (par exemple, la date de dernière modification d'un fichier, ...).

- POST : permet au client d'envoyer des données au serveur, comme par exemple le contenu d'un formulaire renseigné par l'utilisateur. Ces données constituent le corps de la requête.

- Le deuxième champ de la ligne de commande est une URL. Elle désigne en fait la ressource sur laquelle on désire appliquer la commande spécifiée dans le champ précédent. Comme nous l'avons dit à l'instant, cette URL peut aussi bien désigner un fichier statique (HTML, son, ...) ou un programme CGI (Cf. chapitre suivant). Une URL bien formée ressemble à ceci:  	http:// host[:port]/chemin/file.ext

Le numéro de port par défaut est 80.

- Le dernier champ, version, contient la version du protocole HTTP implémenté dans le client considéré. La syntaxe est la suivante : HTTP/version. 	Exemple : HTTP/1.0

- Etudions maintenant l'en-tête associé à la requête exprimée dans la ligne de commande que nous venons de décrire. Il faut tout d'abord savoir que cet en-tête est optionnel. D'ailleurs, une simple requête ne contenant qu'une commande HTTP et une URL est parfaitement utilisable.

Sa structure est la suivante : chaque ligne de l'en-tête comporte un nom de champ (field name), suivi du caractère ":" et d'une valeur (field value). Chaque ligne est séparée de la suivante par les caractères CRLF.

Voici quelques champs fréquemment utilisés dans les requêtes HTTP :

- Content-Encoding : indique l'encodage MIME utilisé par le client dans la requête courante,

- Content-Length : spécifie la longueur du corps de la requête, en octets,

- Content-Type : indique le type d'encodage MIME utilisé pour coder le corps de la requête. Celui utilisé pour transmettre les données html brutes (qui ne sont que du texte) est: Content-Type: text/html

- From : permet d'envoyer au serveur l'adresse E-MAIL définie dans les préférences du navigateur,

- If-Modified-Since : est utilisé pour spécifier une date. Ce champ permet au navigateur de ne demander au serveur l'envoi d'un document que si celui-ci a été modifié depuis cette date.

- User-Agent : permet quant à lui d'indiquer au serveur le nom et la version du navigateur utilisé. Cela peut permettre au serveur d'adapter sa réponse en fonction des caractéristiques du navigateur utilisé.

- Après l'en-tête optionnel, la requête peut contenir un corps, lui aussi optionnel, comportant un certain nombre d'informations dont le format de codage est précisé dans l'en-tête que nous venons de décrire. Le corps ou body, séparé de l'en-tête par une ligne vide, n'est en réalité utilisé que lorsqu'on envoie une requête de type POST.

exemple de requête :

GET /index.html HTTP/1.0 If-Modified-Since : Sunday, 11-May-1997 19:33:11 GMT User-Agent : Mozilla/3.0 (WinNT)

Ici, on demande l'envoi de la page index.html du serveur sur lequel on est connecté à condition que cette page ait été modifiée depuis le 11 Mai 1997. De plus le client transmet des informations concernant son navigateur.

### Réponse HTTP

Etudions maintenant la structure d'une réponse HTTP :

ligne de statut

en-tête

[ligne vide]

corps

- La ligne de statut d'une réponse HTTP comprend trois champs : version    code_réponse    texte_réponse.

- Le premier de ces champs est, comme pour une requête, la version du protocole HTTP utilisé.

- Le deuxième, code_réponse, indique si la requête qui a généré cette réponse a pu être traitée correctement par le serveur.

| Code Réponse | Signification |
| --- | --- |
| 10x | Messages d'information. Non utilisé |
| 20x | Messages indiquant que la requête s'est déroulée correctement |
| 200 | Requête OK |
| 201 | Requête OK. Création d'une nouvelle ressource (commande POST) |
| 202 | Requête OK mais traitement en cours |
| 203 | Requête OK mais aucune information à renvoyer (corps vide) |
| 30x | Messages spécifiant une redirection |
| 40x | Erreur due au client |
| 50x | Erreur due au serveur |

Les codes les plus souvent rencontrés sont : 200, indiquant que la requête s'est déroulée correctement, 304, spécifiant que la page demandée n'a pas été modifiée depuis la dernière consultation et 404, indiquant que la ressource demandée n'existe pas.

- Le troisième, texte_réponse qui correspond à la signification du message ci-dessus est généralement ajouté par le serveur.

- Examinons maintenant l'en-tête d'une réponse. La structure de celui-ci est la même que celle d'une requête. Voici quelques champs que l'on peut rencontrer dans l'en-tête d'une réponse, en sachant que certains de ceux que nous avons vus pour une requête restent valables pour l'en-tête d'une réponse :

- Date : indique la date de génération de la réponse,

- Expires : spécifie la date d'expiration de la ressource demandée,

- Location : contient la nouvelle URL associée au document demandé, lors d'une redirection (codes 30x),

- Server : précise le nom et la version du serveur ayant envoyé la réponse.

Comme pour une requête, le corps de la réponse est séparé de l'en-tête par une ligne vide.

- Le corps ou body contient en fait le document demandé. Cela peut être un fichier HTML simple ou un fichier binaire quelconque, dont le type sera précisé dans l'en-tête par le champ Content-Type.

Exemple de réponse :

HTTP/1.0 200 OK Date : Sunday, 11-May-1999 19:33:14 GMT Server : Apache/1.1 Content-Type : text/html Content-Lenght : 65 Last-Modified : Sunday, 11-May-1999 10:54:42 GMT

<HTML> <body> Bienvenue sur notre site…… </body> /HTML>

Dans cet exemple, le serveur renvoie une page au format HTML, en précisant quelques informations comme la version du logiciel serveur et la date de dernière modification du fichier considéré.

Le langage HTML ne permet pas à lui seul de créer des documents dynamiques ou interactifs capables par exemple d'indiquer la date du jour ou de donner le résultat d'une requête sur une base de données.

La programmation CGI (Common Gateway Interface) a pour but de construire un  document HTML dynamique correspondant à la demande spécifique du client. Le document est envoyé au client au fur et à mesure de sa construction par le serveur.

### Http et CGI

Le client indique le nom d'un fichier à l'aide d'une URL, non pour recevoir son contenu, mais pour demander son exécution au serveur. Ce dernier exécute le programme indiqué et renvoie au client la sortie standard de ce programme (c'est à dire ce que l'on aurait obtenu à l'écran en lançant le programme par une ligne de commande).

Pour pouvoir être exécutés à distance, les exécutables CGI doivent être installés sur le serveur dans un répertoire appelé /cgi-bin. Tout langage ayant une sortie standard peut être utilisé pour écrire des programmes CGI. 

L’interface CGI permet au client d’envoyer au serveur des données saisies par l'utilisateur. Ces données  constituent des arguments d’entrée pour le programme à exécuter. Elles sont codées selon le format suivant:

nom_champ1=valeur1&nom_champ2=valeur2...

où chaque couple (nom du champ, valeur de saisie) est séparé par un '&' et où nom du champ et valeur de saisie sont séparés par un '='.

De plus, cette chaîne est URL-encodée: les espaces y sont remplacés par des '+' et les caractères spéciaux par leur valeur hexadécimale sous le format '%xx'.

Exemple:	NOM=de+la+Maisonneuve&PRENOM=Ren%E9

Une fois la chaîne des données construite, elle est envoyée au serveur selon la méthode GET ou POST de HTTP.

Avec la méthode GET, la syntaxe d’une URL est la suivante:

http://www.site.fr/cgi-bin/nomExecutable?nom=Dupont&pr%E9nom=Ren%E9

Après le mot clef cgi-bin  on trouve le nom du programme exécutable, puis un point d'interrogation "?" suivi de la liste des couples (nom,valeur) encodée comme nous l'avons vu tout à l'heure.

Si on utilise la méthode POST, l’ URL a la syntaxe suivante :

http://www.site.fr/cgi-bin/nomExecutable

et les données sont envoyées dans le corps de la requête HTTP correspondante.

Comparaison :

Avec la méthode GET, les informations sont stockées dans l'URL d'appel du script. Il est alors possible de mémoriser cette URL complète dans un bookmark afin de pouvoir exécuter ultérieurement le script avec un certain nombre d'arguments. Cependant, la longueur des données ainsi transférées est limitée. De plus, il est impossible d'envoyer de cette manière des informations sensibles comme un mot de passe, car elles apparaîtraient en clair à l'écran, dans l'URL...

La méthode POST de HTTP résout les deux problèmes évoqués ci-dessus car aucune information n'est passée via l'URL.

## Carte Lx162

The MVME162LX-222 Embedded Controller is based on the MC68040 microprocessor.

| Feature | Description |
| --- | --- |
| Microprocessor | MC68040 |
| Memory | 4 MB of parity-protected DRAM |
|  | 128 KB of SRAM (with battery backup) |
|  | 1 MB of Flash memory |
|  | Four EPROM sockets |
| Real-time clock | 8KB NVRAM with RTC and battery backup (SGS-Thomson M48T18) |
| Switches | RESET and ABORT |
| Status LEDs | Four: FAIL, RUN, SCON, and FUSES |
| Tick timers | Four programmable 32-bit timers |
| Watchdog timer | Provided in MCchip ASIC (VMEchip2) |
| Serial I/O | EIA-232-D DTE serial interface with four serial ports (Zilog Z85230 controller chips) |
| SCSI I/O | Optional Small Computer Systems Interface (SCSI) bus interface with 32-bit local bus burst Direct Memory Access (DMA) (NCR 53C710 controller) |
| Ethernet I/O | LAN Ethernet transceiver interface with 32-bit local bus DMA (Intel 82596CA controller) |
| IndustryPack Interfaces | 2 IndustryPack Interface sites |
| VMEbus interface | VMEbus requester |
|  | VMEbus system controller |
|  | VMEbus requester |
|  | VMEbus interrupter |
|  | VMEbus interrupt handler |
|  | Eight software interrupts |
|  | Programmable map decoders for the master and slave interfaces |
|  | VMEbus to local bus interface (A24/A32, D8/D16/D32 (D8/D16/D32/D64 BLT) (BLT = Block Transfer) Local-bus-to-VMEbus interface (A16/A24/A32, D8/D16/D32) Two 32-bit programmable Tick Timers and a programmable Watchdog Timer (in the VMEchip2 ASIC) for periodic interrupts |
|  | Global CSR for interprocessor communications |
|  | DMA for fast local memory - VMEbus transfers (A16/A24/A32, D16/D32 ) |

Lx 162 BLOCK DIAGRAM

## IPModule

### introduction

Today there is a clear tendency to buy hardware instead of making it. Arguments that go against this tendency are:

- the board with the functions I need is not available;

- the board is available, but has extra functionality I do not need, making it too expensive.

Mezzanine technology may help in solving this problem. Fundamentally it cuts the problem of desired functions in two separate parts. First decide on the processor to be used, then find a mezzanine carrier board with or without system bus interface and finally, personalise the board by adding one or more mezzanines. If the mezzanine board you are looking for is not available, then it is simpler to make a mezzanine board than to make the carrier board.

By buying the carrier and the mezzanine boards or by making some mezzanine boards yourself you will be quicker in the market with a new design and that is vitally important today.

### IP-Modules

IP is the abbreviation of "Industry Pack". The standard was prepared by VSO (VITA Standards Organisation) .

IP modules exist in two sizes: single size and double size, where the double sized IP module can be seen as two single size IP-modules side by side. The single size module measures 1.8 x 3.9 inch (45.3 cm2), the double 3.6 x 3.9 inch (90.6 cm2).

Components on an IP-module can have a maximum height of 0.29 inch (7.4 mm).

The single size IP-module uses two 50-pin connectors: one for interfacing with the carrier board and one to interface with the external world (I/O via the carrier board).

For example, on a 6U VME carrier board there is place for four single size or two double size IP- modules. Remark that theoretically it is possible to put 6 IP modules on a 6U carrier board, but this is not feasible in practice as all I/O lines have to leave via the carrier board and I/F chips are needed.

IP has no front panel solution for the I/O signals

IP Modules on a 6U VMEbus board

### Bus description

- The IP-bus is a point to point bus from the carrier board to the IP-module. The IP-module is passive on the bus and can only be accessed by the carrier board. (An IP module is always a slave module).

- There is a maximum of 8 MB addressable memory (Memory space mode :22 bit word address [A22 : A1]) and 128 byte I/O space(IO space mode : 6 bit word address [A6 :A1]) per single size IP Module.

- In memory space mode, the IP-bus has a multiplexed address and data bus (D00 : D15 <==>A7 : A22). This reduces the number of pins needed for the mezzanine board but slows down access.

- Auto configuration (for plug and play) is done via an ID PROM.

- 16 bit data transfers can be used on single size boards and 32 bit transfers can be done on double sized boards. There are byte select lines.

- It is a clocked (or synchronous) bus at 8MHz. 32MHz is optional.

- Two interrupt lines are provided per module.

- Two passive DMA channels are possible. This means that a DMA controller function should be on the carrier board if DMA is needed.

As the IP-bus is simple, it is easy to create custom hardware for it.

## Controleur 	IPIC

This chapter describes the IndustryPack Interface Controller (IPIC) ASIC for the MC68040 bus. The IPIC chip is designed for the MVME162LX board and supports up to two IndustryPack (IP) interfaces, designated IP_a through IP_b.

The IPIC chip converts IP-bound MC68040 read/write/interrupt acknowledge cycles to IndustryPack cycles.

### Features

- Provides all logic required to interface the MC68040 bus for two IndustryPacks.

- Supports IndustryPack I/O, Memory, Interrupt Acknowledge, and ID cycles.

- Supports 8-bit, 16-bit, and 32-bit (double size) IndustryPack cycles.

- Provides dynamic bus sizing for accesses to IndustryPack Memory Space.

- Fixed base address for IndustryPack I/O, ID, spaces.

- Programmable base address/size for IndustryPack Memory Space.

- Four Interrupt Handler Control Registers, two for each IndustryPack.

- Recovery timer for each IndustryPack to provide dead time between back to back accesses.

### Clock

The clock speed for the IndustryPack logic interface on MVME162LX 200/300 Series boards is 8MHz.

### Interrupts

The IPIC chip can be programmed to interrupt the local bus master via the IPL* signal pins when one or more of the four  IndustryPack interrupts are asserted. The interrupt control registers allow each interrupt source to be level/edge sensitive and high/low true.

When the local bus master acknowledges an interrupt, if the IPIC determines that it is the source of the interrupt being acknowledged, it waits for IACKIN* to be asserted, then it performs an interrupt acknowledge cycle to the appropriate IndustryPack in order to obtain the vector number. It then passes the vector number on to the local bus master and asserts TA* to terminate the cycle.

When there are multiple IndustryPack interrupts pending at the level being acknowledged, the IPIC performs the interrupt acknowledge for the one with the highest priority. The priority is as follows:

| Interrupt Source PinInterrupt Source Name | Priority |  |
| --- | --- | --- |
| IntReq0 IP_a | IP_a0 | Highest |
| IntReq1 IP_a | IP_a1 | Next Highest |
| IntReq0 IP_b | IP_b0 | Next Lowest |
| IntReq1 IP_b | IP_b1 | Lowest |

### Memory Map

The following memory map table shows devices selected by the IPIC map decoder.

| Address Range | Selected Device | Port Width | Size |
| --- | --- | --- | --- |
| $FFF58000-$FFF5807F | IP_a I/O Space | D16 | 128B |
| $FFF58080-$FFF580BF | IP_a ID Space | D16 | 64B |
| $FFF580C0-$FFF580FF | IP_a ID Space Repeated | D16 | 64B |
| $FFF58100-$FFF5817F | IP_b I/O Space | D16 | 128B |
| $FFF58180-$FFF581BF | IP_b ID Space | D16 | 64B |
| $FFF581C0-$FFF581FF | IP_b ID Space Repeated | D16 | 64B |
| $FFFBC000-$FFFBC01F | Control/Status Registers | D32-D8 | 32B |

### Programming Model

This section defines the programming model for the control and status registers (CSRs) in the IPIC chip. The possible operations for each bit in the CSR are as follows:

R     	This bit is a read-only status bit.

R/W 	This bit is readable and writable.

R/C 	This status bit is cleared by writing a one to it.

C 	Writing a zero to this bit clears this bit or another bit. This bit reads as zero.

S 	Writing a one to this bit sets this bit or another bit. This bit reads as zero.

The possible states of the bits after assertion of the RESET* pin (powerup reset or any local reset) are as defined below.

R 	The bit is affected by reset.

X 	The bit is not affected by reset.

The CSR registers (0xFFFBC000-0xFFFBC01F) can be accessed as bytes, words, or longwords. They should not be accessed as lines. They are shown in the table as bytes, and the bits in the following register descriptions are labeled as bits 7 through 0.

| Offset | Register Name | Register Bit Names |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| $00 | CHIP ID | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| $01 | CHIP REVISION | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $02 | RESERVED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $03 | RESERVED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $04 -$0F | NOT USED |  |  |  |  |  |  |  |  |
| $10 | IP_a INT0 CONTROL | a0_PLTY | a0_E/L* | a0_INT | a0_IEN | a0_ICLR | a0_IL2 | a0_IL1 | a0_IL0 |
| $11 | IP_a INT1 CONTROL | a1_PLTY | a1_E/L* | a1_INT | a1_IEN | a1_ICLR | a1_IL2 | a1_IL1 | a1_IL0 |
| $12 | IP_b INT0 CONTROL | b0_PLTY | b0_E/L* | b0_INT | b0_IEN | b0_ICLR | b0_IL2 | b0_IL1 | b0_IL0 |
| $13 | IP_b INT1 CONTROL | b1_PLTY | b1_E/L* | b1_INT | b1_IEN | b1_ICLR | b1_IL2 | b1_IL1 | b1_IL0 |
| $14 -$17 | NOT USED |  |  |  |  |  |  |  |  |
| $18 | IP_a GENERALCONTROL | a_ERR | 0 | a_RT1 | a_RT0 | a_WIDTH1 | a_WIDTH0 | 0 | a_MEN |
| $19 | IP_b GENERALCONTROL | b_ERR | 0 | b_RT1 | b_RT0 | b_WIDTH1 | b_WIDTH0 | 0 | b_MEN |
| $1A -$1B | NOT USED |  |  |  |  |  |  |  |  |
| $1C | RESERVED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $1D | RESERVED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $1E | RESERVED | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| $1F | IP RESET | 0 | 0 | 0 | 0 | 0 | 0 | 0 | RES |

Chip ID Register

The read-only Chip ID Register is hard-wired to a hexadecimal value of $23. Writes to this register do nothing, however the IPIC terminates them normally with TA*.

| ADR/SIZ | $FFFBC000 (8 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NAME | CID7 | CID6 | CID5 | CID4 | CID3 | CID2 | CID1 | CID0 |
| OPER | R | R | R | R | R | R | R | R |
| RESET | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |

Chip Revision Register

The read-only Chip Revision Register is hard-wired to reflect the revision level of the IPIC ASIC. The current value of this register is $00. Writes to this register do nothing, however the IPIC terminates them normally with TA*.

| ADR/SIZ | $FFFBC001 (8 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NAME | REV7 | REV6 | REV5 | REV4 | REV3 | REV2 | REV1 | REV0 |
| OPER | R | R | R | R | R | R | R | R |
| RESET | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

IP_a, IP_b : IRQ0/IRQ1 Interrupt Control Registers

| ADR/SIZ | $FFFBC010 through $FFFBC013 (8 bits each) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NAME($10) | a0_PLTY | a0_E/L* | a0_INT | a0_IEN | a0_ICLR | a0_IL2 | a0_IL1 | a0_IL0 |
| NAME($11) | a1_PLTY | a1_E/L* | a1_INT | a1_IEN | a1_ICLR | a1_IL2 | a1_IL1 | a1_IL0 |
| NAME($12) | b0_PLTY | b0_E/L* | b0_INT | b0_IEN | b0_ICLR | b0_IL2 | b0_IL1 | b0_IL0 |
| NAME($13) | b1_PLTY | b1_E/L* | b1_INT | b1_IEN | b1_ICLR | b1_IL2 | b1_IL1 | b1_IL0 |
| OPER | R/W | R/W | R | R/W | C | R/W | R/W | R/W |
| RESET | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R |

IL2-IL0

These three bits select the interrupt level for the corresponding IndustryPack interrupt request. Level 0 does not generate an interrupt.

ICLR

In edge-sensitive mode, writing a logic 1 to this bit clears the corresponding INT status bit. In level-sensitive mode, this bit has no function. It always reads as 0.

IEN

When IEN is set, the interrupt is enabled. When IEN is cleared, the interrupt is disabled.

INT

When this bit is high, an interrupt is being generated for the corresponding IndustryPack IRQ. The interrupt is at the level programmed in IL2-IL0.

E/L*

When this bit is high, the interrupt is edge sensitive. When the bit is low, the interrupt is level sensitive.

PLTY

When this bit is low, interrupt is activated by a falling edge/low level of the IndustryPack IRQ*. When this bit is high, interrupt is activated by a rising edge/high level of the IndustryPack IRQ*. Note that if this bit is changed while the E/L* bit is set (or is being set), an interrupt may be generated. This can be avoided by setting the ICLR bit during write cycles that change the PLTY bit. Because IndustryPack IRQ*s are active low, PLTY would normally be cleared.

IP_a, IP_b: General Control Registers

| ADR/SIZ | $FFFBC018 through $FFFBC019 (8 bits each) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NAME($18) | a_ERR | 0 | a_RT1 | a_RT0 | a_WIDTH1 | a_WIDTH0 | 0 | a_MEN |
| NAME($19) | b_ERR | 0 | b_RT1 | b_RT0 | b_WIDTH1 | b_WIDTH0 | 0 | b_MEN |
| OPER | R | R | R/W | R/W | R/W | R/W | R | R/W |
| RESET | ? R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R |

MEN

a_MEN / b_MEN enable the local bus to perform read/write accesses to their corresponding IndustryPack memory space when set, and disable such accesses when cleared

WIDTH1, WIDTH0

The IPIC assumes the memory space data-bus width of  IP_a and  IP_b to be the value decoded from its control bits WIDTH1 and WIDTH0. The following table shows widths inferred by these bits.

| WIDTH1 | WIDTH0 | Memory Space Data Width |
| --- | --- | --- |
| 0 | 0 | 32 bits |
| 0 | 1 | 8 bits |
| 1 | 0 | 16 bits |
| 1 | 1 | reserved |

RT1,RT0

The recovery-timers determine the time that must expire from the acknowledgment of an IndustryPack I/O, ID, or Interrupt Acknowledge cycle until the IPIC asserts a new I/O, ID, or Int SEL* to the same IndustryPack.

| RT1 | RT0 | Recovery Time |
| --- | --- | --- |
| 0 | 0 | 0 microseconds |
| 0 | 1 | 2 microseconds |
| 1 | 0 | 4 microseconds |
| 1 | 1 | 8 microseconds |

ERR

When one of these bits is set to a one, its corresponding IndustryPack Error* signal is asserted.

IP RESET Register

| ADR/SIZ | $FFFBC01F (8 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BIT | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| NAME | 0 | 0 | 0 | 0 | 0 | 0 | 0 | RES |
| OPER | R | R | R | R | R | R | R | S |
| RESET | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R | 0 R |

RES

Setting RES to a one asserts the IPIC IPRESET* signal.

## Module ADA08

Le module ADA 08 de ACTIS Computer permet de réaliser de nombreuses  fonctions avec un simple module IP:

- Système d’acquisition de données 12-bit + signe  avec 8 entrées unipolaires ou 4 entrées différentielles.

- Huit convertisseurs numériques analogiques 12 bits.

- Deux ports  8 bits avec 4 lignes de « handshake ».

- Trois temporisateurs 16 bits.

Can

Cna

Lm12458

8

1

-conv

8

1

Mp7613

8

1

-conv

8

1

Ada08

1

1

-adc

1

1

1

1

-dac

1

1

Port

Compteur

CIOZ8536

1

1

-cio

1

1

2

1

-po

2

1

3

1

-tim

3

1

- Nota : 	Les pages suivantes constituent la documentation constructeur de ce module. Les notices des  composants MP7613, LM12458 et CIO Z8536 ne sont pas nécessaires pour répondre aux questions du sujet d’examen.

Table of contents

1	Introduction	32

2	Summary of features	32

3	Description	33

3.1	Block diagram	33

3.2	IP spaces	34

3.2.1	I/O space	34

3.2.2	ID space	35

3.2.3	Wait state cycles	36

4	Characteristics	36

4.1	CIO	36

4.2	DAC	36

4.3	ADC	37

5	Zilog CIO Z8536	38

5.1	General-purpose ports	38

5.2	Special-purpose port	38

5.3	Counter/Timers	39

5.4	CIO Interrupt function	39

5.5	Interrupt timing	39

5.6	Z8536 registers access	40

6	MPS MP7613	43

6.1	DAC code registers (offset $40 to $4E)	43

6.2	DACREG: Output Update and reset register (offset $79)	44

6.3	Conversion codes	44

6.4	Precision reference voltage	44

7	LM12H458 acquisition system	45

7.1	Interrupt vector (ADIV) register (offset $61)	45

7.2	Enable ADC interrupt (EADI) register (offset $71)	45

7.3	External synchronisation	46

7.4	Conversion codes	46

7.5	Precision reference voltage	46

7.6	Voltage range selection	46

8	Connection	47

9	Component location	48

Appendix : Program examples	49

Although the information contained in this document is believed to be correct, ACTIS Computer S.A. can not be held responsible for any error or for any resulting consequential losses.

ACTIS Computer S.A may change or improve the specifications of its products at any time without prior notification.

 Copyright ACTIS Computer S.A.	 1997

# Introduction

The ADA-08 IP module of ACTIS Computer S.A. represents a highly integration on a single size IP, with support for a wide range of digital and analog I/O, divided in three sections :

o  	A counter/timer and parallel I/O unit (CIO). This section provides

two  general-purpose 8-bit ports, one special-purpose 4-bit ports, and     three 16-bit counter/timers.

o  	An octal 12-bit digital to analog converter (DAC). This function

provides eight single-ended buffered analog outputs in bipolar modes, which will directly accept, to drive an analog load on the DAC outputs.

o  	A 12-bit+sign analog to digital converter (ADC). This function

provides eight single-ended or four differential analog inputs for unipolar signals. An on-chip 8-word RAM can store the conversion sequence for up to eight acquisitions through the on-chip eight-input multiplexer.

The ADA-08 is compatible with the VITA-4 IP module specification.

# Summary of features

- Single-size IP module.

- Compatible with the VITA-4 IP Module specification.

- Two general-purpose 8-bit ports.

- One special-purpose 4-bit ports.

- Three 16-bit counter/timers.

- Flexible pattern-recognition logic for interrupt generation.

- All signals in TTL levels.

- Octal 12-bit  digital to analog conversion.

- Integral non linearity of ±2 LSB.

- Settling time less than 35 µs.

- On-board output amplifiers.

- Output voltage range: ±10 V

- Octal 12-bit + sign analog to digital converter.

- Up to 140 kSample/s throughput.

- On-chip multiplexer and instruction sequencer.

- Conversion accuracy of ±1 LSB.

- Settling time less than 5 µs.

- Single-ended and differential input modes.

- Input voltage range : ±10V (standard version) 
Factory option : ±5 V, 0 to +10 V and 0 to +5 V (min quantity required)

- Monitoring functions.

- Auto calibration and diagnostic mode.

- On-chip 32-word FIFO.

- External trigger capability.

- On-board two +5 V precision voltage reference.

- Industrial temperature range option (-40 to +85 °C).

# Description

The CIO section of ADA-08 contains two general-purpose 8-bit ports, one special-purpose   4-bit and three 16-bit counter/timers.

All  I/O signals, TTL levels, are directly connected from CIO controllers to the IP 50-pin I/O connector.

The DAC function is designed  on the MP7613 (12-bit) digital to analog converter. Each DAC output is followed by an analog amplifier, to directly drive an analog load. Each output voltage range is  ±10 V.

The modules includes a precision voltage reference reference (25 ppm/°C maximum temperature drift), factory calibrated, for digital to analog conversion and analog acqisition.

The ADC function is based on the LM12H458 analog to digital converter from National Semiconductor. This device offers very useful capabilities for acquisition operations: monitoring functions, on-chip FIFO, auto-calibration and diagnostic mode, programmable acquisition time and conversion rates. This device also provides a sequencer, which executes up to eight instructions. Each instruction defines the internal multiplexer selection, the conversion resolution, the acquisition time and the timer capability.

The ADC function provides a voltage range for each analog input between ±10 V (standard version).

(Factory option for min qty: ±5 V, 0 to +10 V and 0 to +5 V ).

The LM12H458 also provides single-ended and differential input modes.

## Block diagram

The hardware architecture of the ADA-08 is divided in six general  main sections:

- The IP bus interface, controlling the timing access of the peripheral controllers.

- The CIO controller, Zilog Z8536.

- The  MP7613 digital to analog converter.

- The LM12H458 analog to digital converter.

- The analog output amplifiers.

- The analog input amplifiers and optionally low pass filter.

## IP spaces

The following table gives the four ADA-08 memory spaces.

| IP space | Devices |
| --- | --- |
| MEMORY | No access |
| IDENTIFICATION | IP identification codes |
| I/O | - CIO registers - DAC Data registers - DAC Control register - ADC Data registers - ADC control register - ADC Interrupt vector register - ADC Enable interrupt vector register |
| INTERRUPT ACK. | CIO Interrupt acknowledge cycle ADC Interrupt acknowledge cycle |

The base address of these spaces depends on the IP carrier.

### I/O space

The CIO controller is mapped into the IP I/O space. Data and command registers of the CIO controller are placed on 4 consecutive addresses (8-bit data path).

| I/O space adr. | Register | Description |
| --- | --- | --- |
| CIO controller |  |  |
| $51 | PCDR | Port C Data Register |
| $53 | PBDR | Port B Data Register |
| $55 | PADR | Port A Data Register |
| $57 | CR0 | Control Register 0 |

All port configurations are defined with the CIO control registers. The data registers allow the value of each ports to be accessed directly.

The DAC converter is mapped into the IP I/O space and provides eight 16-bit data registers and one 8-bit command registers.

| I/O space adr. | Access | Bus wide | Register | Description |
| --- | --- | --- | --- | --- |
| $40 | R/W | 16-bit | DAC0 | DAC code for channel 0 |
| $42 | R/W | 16-bit | DAC1 | DAC code for channel 1 |
| $44 | R/W | 16-bit | DAC2 | DAC code for channel 2 |
| $46 | R/W | 16-bit | DAC3 | DAC code for channel 3 |
| $48 | R/W | 16-bit | DAC4 | DAC code for channel 4 |
| $4A | R/W | 16-bit | DAC5 | DAC code for channel 5 |
| $4C | R/W | 16-bit | DAC6 | DAC code for channel 6 |
| $4E | R/W | 16-bit | DAC7 | DAC code for channel 7 |
| $79 | W | 8-bit | DACREG | DAC Output Update and reset register |

The LM12458 interrupt vector registers are mapped into the IP I/O space.

| I/O space adr. | Access | Bus wide | Register | Description |
| --- | --- | --- | --- | --- |
| LM12H458 registers |  |  |  |  |
| $00 | R/W | 16-bit | RAM0 | Instruction RAM0 |
| $04 | R/W | 16-bit | RAM1 | Instruction RAM1 |
| $08 | R/W | 16-bit | RAM2 | Instruction RAM2 |
| $0C | R/W | 16-bit | RAM3 | Instruction RAM3 |
| $10 | R/W | 16-bit | RAM4 | Instruction RAM4 |
| $14 | R/W | 16-bit | RAM5 | Instruction RAM5 |
| $18 | R/W | 16-bit | RAM6 | Instruction RAM6 |
| $1C | R/W | 16-bit | RAM7 | Instruction RAM7 |
| $20 | R/W | 16-bit | CONF | Configuration register |
| $24 | R/W | 16-bit | ENINT | Enable Interrupt register |
| $28 | R | 16-bit | INTST | Interrupt Status register |
| $2C | R/W | 16-bit | TIMER | Timer register |
| $30 | R | 16-bit | FIFO | Conversion FIFO |
| $34 | R | 16-bit | LIMIT | Limit status register |
| $61 | R/W | 8-bit | ADIV | ADC Interrupt vector register |
| $71 | R/W | 8-bit | EADI | Enable ADC Interrupt register |

### ID space

The identification space is defined as follows:

| ID space adr. | Description | value |
| --- | --- | --- |
| $01 | Ascii "I" | $49 |
| $03 | Ascii "P" | $50 |
| $05 | Ascii "A" | $41 |
| $07 | Ascii "C" | $43 |
| $09 | Manufacturer identification | $99 |
| $0B | Module type | $24 |
| $0D | Revision module |  |
| $0F | Reserved | $00 |

 See below for description.

The four first bytes contain the ASCII text "IPAC". This clearly identifies the ROM beginning.

The manufacturer code $99 identifies ACTIS IP.

The ADA-08 device is defined by the module type byte $24.

The next byte identifies the IP revision, in accordance with the following definition.

| Revision | ASCII car. | hex. value |
| --- | --- | --- |
| First | "_" (space) | $5F |
| Next | "A", "B", ... | $41, $42, ... |

The byte $0F is reserved for future extension.

### Wait state cycles

The following tables gives the number of wait states asserted in each IP space.

| Space | Wait state |  |
| --- | --- | --- |
|  | Read | Write |
| Identification | 0 | N.A. |
| I/O CIO o CIO registers o Interrupt acknowledge | 4 5 | 4 N.A. |
| I/O ADC o ADC registers o Interrupt Vector (IV) register o Enable interrupt Vector (EIV) register o Interrupt acknowledge | 2 0 0 0 | 2 0 0 N.A. |
| I/O DAC o DAC data registers o DACOU register o DACRST register | 6 N.A. N.A. | 2 2 0 |

# Characteristics

## CIO

| Symbol | Parameters | Test conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VIH | Input high voltage |  | 2 | Vcc + 0.3 | V |
| VIL | Input low voltage |  | -0.3 | 0.8 | V |
| VOH | Output high voltage | IOH=-250 µA | 2.4 |  | V |
| VOL | Output low voltage | IOL=+2 mA IOL=+3.2 mA |  | 0.4 0.5 | V |
| ICC | Vcc supply current |  |  | 310 | mA |
| PCLK | Peripheral clock |  |  | 4 | MHz |

## DAC

| Symbol | Parameters | Test conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| N | Resolution |  |  | 12 | bit |
| INL | Integral non linearity | Ta=25 °C |  | ±2 | LSB |
| DNL | Differential non linearity | Ta=25 °C |  | ±1 | LSB |
| ZOFS | Bipolar zero offset | Ta=25 °C |  | ±2 | LSB |
| _E | Channel-to-channel max. error (all code) | Ta=25 °C |  | ±1 | LSB |
| ME | All channels maximum error | Ta=25 °C |  | ±4 | LSB |
| MES | All channel error span | Ta=25 °C |  | 5 | LSB |
| sd | Settling time from DAC output writing to DAC output valid | Zero to Full scale. (20 V step) |  | 55 (typ. 35) | µs |
| Symbol | Parameters | Test conditions | Min | Max | Unit |
| Rload | Analog load | ±10 V range ±5 V range 0 to +10V range | 2 400 2 |  | kW W kW |
| TCVo | Voltage reference temperature coefficient |  |  | 25 | ppm/°C |
| Vref | Voltage reference | FS DAC0 min. error |  | typ. 5 | V |
| Vcc | Vcc analog voltage |  | 11.4 | 12.75 | V |
| Vee | Vee analog voltage |  | -12.75 | -11.4 | V |
| Vdd | Vdd digital voltage |  | 4.5 | 5.5 |  |
| Icc | Positive supply current |  | 90 | 210 | mA |
| Iee | Negative supply current |  | 90 | 210 | mA |
| Idd | Digital supply current |  | 95 | 120 | mA |

## ADC

| Symbol | Parameters | Test conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| N | Resolution | After auto-cal. |  | 12+sign | bit |
| INL | Integral non linearity | After auto-cal. |  | ±1.5 | LSB |
| DNL | Differential non linearity | After auto-cal. |  | ±1 | LSB |
| ZOFS | Bipolar zero offset | After auto-cal. |  | ±1.5 | LSB |
| FSE | Full-scale error | After auto-cal. |  | ±2 | LSB |
| Tc | Conversion time | 12-bit + sign 9-bit + sign |  | 11 5.3 | µs |
| Tc | Conversion time | 12-bit + sign 8-bit + sign |  | 11 5.3 | µs |
| Tacq | Acquisition time | 12-bit + sign 8-bit + sign |  | 2.3 0.6 | µs |
| Tz | Auto-zero time | Sequencer S2 |  | 19 | µs |
| Tcal | Full calibration time | Sequencer S2 |  | 1.2 | ms |
|  | Throughput rate | Standard version. |  | 140 | kS/s |
| sd | Settling time, analog interface propagation delay. | 10 V step |  | 2 | µs |
| ZIN | Impedance input on input multiplexer |  | 10 |  | kW |
| Rs | Source resistance to LM12458 converter |  |  | < 10 | W |
| TCVo | Voltage ref. temp. coefficient |  |  | 25 | ppm/°C |
| CLK | LM12458 clock | Standard version |  | typ . 8 | MHz |
| Vref | Voltage reference |  |  | typ. 5 | V |
| Vcc | Vcc analog voltage |  | 11.4 | 12.75 | V |
| Vee | Vee analog voltage |  | -11.4 | -12.75 | V |
| Vdd | Vdd digital voltage |  | 4.5 | 5.5 | V |
| Icc | Positive quiescent current |  |  | 50 | mA |
| Iee | Negative quiescent current |  |  | 40 | mA |
| Idd | Digital quiescent current |  |  | 180 | mA |
| Ta | Operating ambient temperature | Standard version | 0 | +70 | °C |
| Tstg | Storage temperature |  | -60 | +150 | °C |

# Zilog CIO Z8536

The ADA-08 IP contains a Z8536 devices, and the following description gives you the features of these CIO controller.

The Z8536 device provides two general-purpose 8-bit ports (port A and B), one special-purpose 4-bit port (port C), and three 16-bit counter/timers (timer 1 to 3).

The CIO controller provide an interrupt IRQ0 connected to IntReq0 of the IP logic interface.

## General-purpose ports

These ports can be programmed either as a handshake-driven, and as single or as double buffered port (input, output, or bi-directional) with the direction of each bit individually programmable. These two ports (ports A and B ) can also be linked to form a 16-bit I/O port with handshake. Each port can generate an interrupt when a specific pattern-recognition is detected. This pattern offers a very large flexibility for a specific OR/AND condition, the polarity and the transition detection (state or edge) are independently programmable for each bit-port.

The second port (port B) allows an external access to the first and second software counter/timers (counter/timers 1and 2).

To program these capabilities, both ports contain 12 registers. The command and status registers define the primary port functionality. Two registers are used to define the port specification and handshake modes. The I/O data is accessed through the input, output and buffer registers. The data direction, data path polarity and special I/O control registers allow programming each bit. The pattern-recognition is defined with the pattern polarity, pattern transition and pattern mask registers.

## Special-purpose port

The function of this port (port C) is defined in accordance with the role of the general-purpose ports. This port can provide either handshake lines for the general-purpose ports, either an external access to the third counter/timer (counter/timers 3) or either conventional I/O lines.

Only three registers (data path polarity, data direction, and special I/O control) are necessary to control the port C when the general-purpose ports functionality (ports A and B) have been specified.

## Counter/Timers

All the three counter/timers (counter/timers 1 to 3) are functionality identical.

Three output modes are available: pulse, one-shoot, and square-wave. The operating mode of the counter/timer can be programmed either as retriggerable or as non-retriggerable.

Up to four control lines are provided for timer/counter operation : counter input, gate input, trigger input, and counter/timer output.

Each counter/timer provides a time-constant register containing the initial value of the down-counter. At any time the current count register shows the content of the down-counter. Two other registers are used the control register to set the timer/counter configuration, and the timer/counter status register.

## CIO Interrupt function

Five registers control the CIO interrupt functions: the master interrupt control, three interrupt vector registers, and the current interrupt vector register.

## Interrupt timing

The following timing descriptions give the delay for interrupt generation.

In bit port mode, an interrupt is generated when the pattern-recognition is detected.

In handshake port operation, the external ACKIN signal strobes the input data into the CIO and an interrupt is activated to indicate to the host that a valid input data is present.

| Symbol | Parameters | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| TwPM | Pattern match input valid (bit port) | 750 |  | ns |
| TdPM | Pattern match to INT delay (bit port) |  | 1.3 | µs |
| TdACK | ACKIN to INT delay (port with handshake) |  | 3.1 | µs |
| Td | Counter input to INT delay (timer mode) |  | 1.2 | µs |

## Z8536 registers access

Two cycles are necessary to address an internal register, as the internal CIO registers are accessed through an internal pointer register. The first cycle is the write pointer operation, into the control register (CR0) which select the internal register.

The next cycle is the data access operation.

The following state machine shows the internal register operation.

Hardware

or software

RESET

WR

Pointer

register

The CIO is on the reset state, after hardware or software reset. The software reset is accomplished by writing D0=1 in CR0 register. On the CIO controller, the hardware reset is accomplished when the reset signal is asserted on the IP logic interface. To  leave this state, an initialisation cycle is necessary: write D0=0 in the control register, otherwise during a read cycle the CIO device returns always D0=1. The state 0 waits for the pointer writing (in the control register) for the internal register selection. After the data access (read or write cycle), the state machine returns to the state 0, waiting for the next pointer writing.

A minimum time of 500 ns is required between the pointer writing and the data access operation.

All internal CIO registers, including the internal data register, are accessed through this method. The pointer register is automatically cleared after each following read or write cycle.

A direct register access, without pointer writing cycle, is available for the data register with the PADR to PDDR registers (see I/O space section).

The following table shows the internal CIO registers.

| CIO Internal addr. | Cycle | Registers |
| --- | --- | --- |
| Main control registers |  |  |
| $00 | R/W | Master interrupt control |
| $01 | R/W | Master configuration control |
| $02 | R/W | Port A interrupt vector |
| $03 | R/W | Port B interrupt vector |
| $04 | R/W | Counter/timer interrupt vector |
| $05 | R/W | Port C data path polarity |
| $06 | R/W | Port C data direction |
| $07 | R/W | Port C special I/O control |
| Most often accessed registers |  |  |
| $08 | R/partial W | Port A command and status |
| $09 | R/partial W | Port B command and status |
| $0A | R/partial W | Counter/Timer 1 command and status |
| $0B | R/partial W | Counter/Timer 2 command and status |
| $0C | R/partial W | Counter/Timer 3 command and status |
| $0D | R/W | Port A data |
| $0E | R/W | Port B data |
| $0F | R/W | Port C data |
| Port A specification registers |  |  |
| $20 | R/W | Port A mode specification |
| $21 | R/W | Port A handshake specification |
| $22 | R/W | Port A data path polarity |
| $23 | R/W | Port A data direction |
| $24 | R/W | Port A special I/O control |
| $25 | R/W | Port A pattern polarity |
| $26 | R/W | Port A pattern transition |
| $27 | R/W | Port A pattern mask |
| Port B specification registers |  |  |
| $28 | R/W | Port B mode specification |
| $29 | R/W | Port B handshake specification |
| $2A | R/W | Port B data path polarity |
| $2B | R/W | Port B data direction |
| $2C | R/W | Port B special I/O control |
| $2D | R/W | Port B pattern polarity |
| $2E | R/W | Port B pattern transition |
| $2F | R/W | Port B pattern mask |
| Counter/Timer related registers |  |  |
| $10 | R | Counter/Timer 1 current count MSB |
| $11 | R | Counter/Timer 1 current count LSB |
| $12 | R | Counter/Timer 2 current count MSB |
| $13 | R | Counter/Timer 2 current count LSB |
| $14 | R | Counter/Timer 3 current count MSB |
| $15 | R | Counter/Timer 3 current count LSB |
| $16 | R/W | Counter/Timer 1 time constant MSB |
| $17 | R/W | Counter/Timer 1 time constant LSB |
| $18 | R/W | Counter/Timer 2 time constant MSB |
| $19 | R/W | Counter/Timer 2 time constant LSB |
| $1A | R/W | Counter/Timer 3 time constant MSB |
| $1B | R/W | Counter/Timer 3 time constant LSB |
| $1C | R/W | Counter/Timer 1 Mode Specification |
| $1D | R/W | Counter/Timer 2 Mode Specification |
| $1E | R/W | Counter/Timer 3 Mode Specification |
| $1F | R | Current vector |

For more details on register programming and SCC functionality, please refer to the Zilog "Z8036 Z-CIO/Z8536 CIO Counter/Timer and Parallel I/O Unit" technical manual.

# MPS MP7613

The IP modules work with octal DAC devices from Micro Power System to achieve to digital to analog conversion. One of these devices is used as per the conversion resolution required. The MP7613 converts in 12-bit resolution.

The internal architecture of these devices is divided in two latch sections. The first contains eight 16-bit data registers (first latch-bank, LB0 to LB7 registers), which store the DAC conversion codes. These registers may be individually accessed for each DAC output. However, the relevant DAC analog output is not updated when a new value is writing. The DAC Output Update register (DACREG, bit D1) allows the transfer, in a single cycle, of the content in the eight first latches (LB0 to LB7 registers) into the second latch-bank (LB0' to LB7') and update the analog output.

The reset DAC (DACREG register) is used to execute a general reset cycle into the converter (see the 6.2. section).

An on-board precision voltage reference of +5 V provides a very high temperature stability of 25 ppm/°C drift to the DAC converter.

## DAC code registers (offset $40 to $4E)

The eight independent, read or write, DAC code registers contain the conversion codes for each analog output. The data bits are aligned to the bit D2 (12 bit, D2 to D13). When a new value is stored, the corresponding output is not affected. To update the analog output , it is necessary to make a write cycle in the DACREG register (see the following section).

DAC code format

| IP-BUS 16-bit | D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAC 12-bit | - | - | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | - | - |

- : Undefined.

Note that the bits D[15 :14 ; 1-0] are unused.

## DACREG: Output Update and reset register (offset $79)

This register transfers the first latch-bank content into the second latch-bank, which are directly connected to the MP7613 internal output amplifiers. In general application, the eight DAC code registers are set in a first step and then the DACREG operation is executed. At that time, only the required DAC outputs are changed, without alteration for the other not concerned outputs.

To execute this operation, the D1 bit of DACREG register (Update bit) must be set to 1.

To execute a reset operation, the D0 bit of DACREG register (Reset bit) must be set to 1.

| IP-Bus | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DACREG | - | - | - | - | - | - | Update | Reset |

- : Undefined.

The following table shows the DAC states after a reset cycle.

|  | 12-bits du DAC |
| --- | --- |
| First latch value | $0800 |
| Second latch value | $0800 |
| Analog output voltage ±10 V range | 0 Volt |

Note that the DAC function execute also a hardware reset at each system reset.

## Conversion codes

The following equation gives the analog output voltage according to the conversion code (D parameter) for a 12-bit conversion in 10V analog output ranges.

| DAC 12-bit converter |  |
| --- | --- |
| Output range | Conversion code |
| ±10 V | 10 * (-1 + 2 * D / 212 ) |

## Precision reference voltage

The ADA-08 IP module provides a voltage reference for digital to analog conversion. It offers a very good temperature stability, less than 25 ppm/°C rift. It is factory calibrated, with a potentiometer, to give to DAC0 a minimum total error, in full scale conditions.

# LM12H458 acquisition system

The ADC function works with the enhanced LM12H458 converter. This highly integrated data acquisition system is a 12-bit + sign with integrated sample-and-hold. This acquisition system operates in three different modes: 12-bit + sign, 8-bit + sign, and watchdog. The watchdog function generates an interrupt request when the input goes out from the range defined by the two-limit references. The acquisition and conversion times are software programmable with the on-chip timers.

An internal 32-word FIFO reduces the CPU overhead. When this FIFO is used, it is possible to stores up to 32 consecutive conversions before reading the FIFO values.

The LM12H458 has an auto-calibration mode, which executes an auto-zero offset voltage correction and an ADC linearity calibration with eight samples of the offset voltage. A short auto-zero correction is also provided for every conversion.

A synchronisation signal SYNC is present to operate either as an input or an output. When the SYNC signal is programmed as an input, a rising edge applied to this pin starts a conversion cycle or a watchdog comparison.

The LM12458 internal architecture includes a sequencer, which executes programmable instructions. The instruction RAM holds up to eight executable instructions. The conversion modes, sequencer operation (continue or single), synchronisation capability, conversion resolution and the timer functions are defined into this instruction RAM.

One option with the "Sync" signal wired to the bus "Strobe" line is also available.

Recommended initialisation:

The following steps must be achieved to insure correct analog conversions.

Execute a full LM12H458 auto-calibration through the CONF register.

For more details on register programming and ADC functionality, please refer to the National Semiconductor "LM12454/ LM12H454/ LM12458/ LM12H458 12-bit + sign data acquisition system with self-calibration" data sheets.

## Interrupt vector (ADIV) register (offset $61)

This read and write ADIV register includes the 8-bit vector for the interrupt acknowledge cycle, generates in response of a LM12458 interrupt request.

| IP-Bus | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADIV | ADIV7 | ADIV6 | ADIV5 | ADIV4 | ADIV3 | ADIV2 | ADIV1 | ADIV0 |

## Enable ADC interrupt (EADI) register (offset $71)

The LM12H458 interrupt is enable  when the D0 bit (enable bit) of read and write EAI register  must be set to 1.

| IP-Bus | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EADI | - | - | - | - | - | - | - | Enable |

- : Undefined

## External synchronisation

The ADA-08 provides, on the 50-pin I/O connector interface, an external synchronisation signal (input or output). The SYNC signal controls the LM12H458 start conversion, and allows the synchronisation with other converter modules in multiple acquisitions system.

The synchronisation signal from the I/O connector is directly connected from the LM12H458 device, and detects a rising edge.

(One option with the "Sync" signal wired to the bus "Strobe" line is also available.)

## Conversion codes

The following table gives the conversion codes for the 12-bit + sign and 
8-bit + sign resolutions.

| Voltage range | Conversion codes |
| --- | --- |
| ±10 V | VIN+1020⋅N-1 |
| ±5 V * | VIN+510⋅N-1 |
| 0 to +5 V * | VIN5⋅N-1 |
| 0 to +10 V * | VIN10⋅N-1 |

* Factory options

The N parameter defines the acquisition resolution for the analog to digital conversion (4096 for 12-bit, 256 for 8-bit conversions).

## Precision reference voltage

The ADA-08 module provides a second voltage reference for the analog to digital conversion. It offers a very good temperature stability, less than 25 ppm/°C drift. It is factory calibrated, with a potentiometer, in order to give the minimum full scale error.

## Voltage range selection

The input voltage is ±10 V on the standard version.

The bipolar ±5 V, unipolar 0 to 5 V and 0 to 10 V are also available as a factory option or special order with minimum quantity(please contact ACTIS Computer marketing dpt).

# Connection

The ADA-08 signals of the two general-purpose 8-bit ports and the  special-purpose 4-bit ports, the eight analog output, and the eight single-ended or the four differential analog input are present on the 50-pin IP I/O connector. The power supply (+5V and ±12V) is also provided for external signal conditioning, and it's short-circuit protected.

| Signals | Description |
| --- | --- |
| PA[7:0] | Port A general-purpose 8-bit port |
| PB[7:0] | Port B general-purpose 8-bit port |
| PC[3:0] | Port C special-purpose 4-bit port |
| EVO[7 :0] | DAC output [7 :0] |
| EVI[7 :0] | ADC input[7 :0] |
| SYNC | ADC External synchro. |

| POWER |  |
| --- | --- |
| Signals | Description |
| +12 V | +12 V, 500 mA max. fuse protected. |
| -12 V | -12 V, 500 mA max. fuse protected. |
| VDD | + 5 V, 500 mA max. Fuse protected. |
| GND | Digital ground |
| AGND | Analog ground |

# Component location

Component side:

Back side :

Although the information contained in this document is believed to be correct, ACTIS Computer S.A. can not be held responsible for any error or for any resulting consequential losses. ACTIS Computer S.A. may change or improve the specifications of its products at any time without prior notification.

Appendix : Program examples

To use the DAC MP7613 (page 43) converter:

This program shows how to use the DAC converter. The value is placed in the DACx register (defined by the _dac structure) and the output is enable writing a 0x2 on the DACREG register.

void	main (void)

{

short int		value, code;

struct _dac		*dac_ptr;

unsigned char	*dacreg_ptr;

dac_ptr		= (struct _dac*) 0x4200040;	/* 0x420 0000 = ad base Ada08 */

dacreg_ptr	= (unsigned char*) 0x4200079;

code 	= 4*0x000;			/* 12 bits converter: LSB = bit2	*/

dac_ptr->dac_0 = code;		/* channel 0 : output = -10 volts */

code = 4*0x800;

dac_ptr->dac_1 = code;		/* channel 1 : output = 0 volts	*/

code = 4*0xfff;

dac_ptr->dac_2 = code;		/* channel 2 : output = +10 volts */

*dacreg_ptr = 0x2;		/* enable output */

}

To use the ADC LM12458 (page 45) converter:

This program shows how to use the ADC converter. First, calibrate the ADC with functions auto-zero and full scale. Second, initialize the RAMx register with the conversion mode. Third, start conversion and forth, read values in the FIFO register. The _adc structure define ADC registers.

void	main (void)

{

volatile int		i;

struct _adc		*adc_ptr = (struct _adc*) 0x4200000;

short int		adc_value[8];

adc_ptr->conf_reg = 0x2;			     /* reset */

adc_ptr->conf_reg = 0x4;			     /* auto-zero */

while ((adc_ptr->interr_stat & 0x8) != 0x8);     	     /* check if auto-zero has  been completed */

adc_ptr->conf_reg = 0x8;			     /* Full scale */

while ((adc_ptr->interr_stat & 0x10) != 0x10);       /* check if full scale has been completed */

adc_ptr->ram_0 = 0x2000 + 2;	 		    /* acquisition time=2 / pause bit */

adc_ptr->ram_1 = 0x2000;			    /* 8 acquisitions sequence preparation */

adc_ptr->ram_2 = 0x2000;

adc_ptr->ram_3 = 0x2000;

adc_ptr->ram_4 = 0x2000;

adc_ptr->ram_5 = 0x2000;

adc_ptr->ram_6 = 0x2000;

adc_ptr->ram_7 = 0x2000;

adc_ptr->conf_reg = 0x2;					/* reset */

adc_ptr->conf_reg = 0x1;					/* start */

while ((adc_ptr->interr_stat & 0x4000) != 0x4000);		/* check end conversion */

for(i=0; i<7; i++)     adc_value[i] = adc_ptr->fifo_reg;

}

| Structures: struct _dac { short int dac_0; short int dac_1; short int dac_2; short int dac_3; short int dac_4; short int dac_5; short int dac_6; short int dac_7; }; | struct _adc { short int ram_0; short int rsvd8; short int ram_1; short int rsvd9; short int ram_2; short int rsvd10; short int ram_3; short int rsvd11; short int ram_4; short int rsvd12; short int ram_5; short int rsvd13; short int ram_6; short int rsvd14; short int ram_7; short int rsvd15; short int conf_reg; short int rsvd16; short int ena_int; short int rsvd17; short int interr_stat; short int rsvd18; short int tim_reg; short int rsvd19; short int fifo_reg; short int rsvd20; short int lim_stat; }; |
| --- | --- |